Boot upgrade instruction for 2700A model from current b2 to b75.

- b75 boot upgrade needed only one time for each receiver to make enough space for future software upgrade.

- Connect null modem cable between PC and receiver.

- Open new GTRom Loader v27.02(Pansat2700A B75). Old version v27.01 will not works.

- Click BIN File and open “Boot_2700A(b75)_(07)0310” new boot file.

- Turn the receiver power “ON” and click download and wait until “Download Completed OK” appears. Front LED will display moving b-7-5 left to right if upgraded succeeded.

- Open new bin file matching with b75 boot version (Any api version dated on or after Mar 10, 2007) and click download. Wait until “Download Completed OK” appears. Front LED will display “ON” or any channel number after b-7-5 shows shortly if download succeeded.

- 2700A, 2700A(IR), 2700E all 3 models can be applied.

WARNING: Do not interrupt the main power until download completed OK appears on the PC screen. It may damage the receiver seriously. If download fail appears, check the api bin file date if it is old version before Mar 10, 2007. It should be Mar 10, 2007 or after dated one.