FTLink User Guide for LAVA2600 Receiver

# Steps to be followed for upgrading

### Connect receiver to your PC using RS232C (cross or null) cable

Using a twisted/cross serial cable connect the PC with the RS232 port of the receiver. Before connecting make sure that the cable is working.

2. Turn on the Receiver and check the Front Display is “boot”

When the receiver is powered ON there will be a display “boot” in the front panel LCD.

3. Run FTLINK in your PC, then the upgrade main menu will be started

From the PC run the FTLINK.exe file. Upon executing the exe the main menu of the FTLink will be displayed.

4. Press “Setup Comm” to select Port connected to Receiver

By pressing the “Setup Comm” option in the FTLink menu, select which COM port is connected to the PC.

## Software Upgrade

1. If correct Port is selected, then the information of current version of the Receiver will be displayed on the main menu.

2. If you want to upgrade now, then press “Upgrade Software” and find the “rom.bin” file to be up-graded in your PC. Then press “Open”.

3. During the download, your PC and Front panel of Receiver will display the procedure situation.

4. After completed Download, Receiver automatically writes downloaded data in its Flash Rom. Please wait until display “GOOD” message on the Front Panel Display. Make sure that the seven-segment display shows “GOOD” message. When there is a problem in the download the seven-segment display will show an error message.  Power off your receiver and turn on again.

- Turn on the keycode

After you upgrade to the new BIN, go to the Installation Menu, select System Setting, On the system setting screen, use your remote and enter Key “4132”, you will see a red button appears at the left corner of the screen. Now the key function is activated. You can use the red button(audio) on the remote to enter the key you know also.

## This BIN has auto load function and it takes 3-10 minutes for the receiver to match the correct key for the first time you upgrade your receiver.

## Read User Data

1. If you select Read User Data button. You make any file, you can read your receiver data. This receiver data will contain all the channels that are tuned by the receiver. The output will be a text file and you can also edit in your PC.

## Write Factory Data

1. If you made your own data, you can upgrade in your receiver with this button.

2. If you want to write in your receiver, press “write factory data” button.

3. Select your own file.

4. During the download, your PC and Front panel of Receiver will display the procedure situation.

5. After completed Download, Receiver automatically writes downloaded data in its Flash Rom. Please wait until display “GOOD” message on the Front Panel Display.

6. And plug off and On.

Good luck and Enjoy…