Viewsat 2000 Ultra Setup Guide Using Serial Cable (NULL)/USB

- Loading your BIN using Null cable:

- Connect serial cable (NULL) to PC>STB

- Open up Loader of choice (PVR-Loader 1.1 or Loader2)

- Click on Tool>Configuration>Com port (Default is com 1)

- Click on Open File

- Click on PC>STB (Notice the progression bar in loader)

- Turn the power in the back ON

- Wait for Loader say Completed.  Then on box LCD front panel screen say I.N.S.T>END>0000 or until clock count to 0.001

IT IS VERRY IMPORTANT. DO NOT TURN THE POWER OFF TO SOON

- Turn the power OFF in front and power OFF in back

- After downloading the  “BIN”, unhook NULL cable from PC>STB

- Connect all the cables to your box/TV

- Turn the power ON in the front and back

- Press Menu key

- Select Installation and press OK

- Select Antenna Setup and press OK

- Select your Sat.

- Here if you have a DISEqC switch enter the port #, Motor or Legacy SW

- No LNB Type need to select (from new, you have only two selection Standard and Universal).  STB will select your LNB Automatic

- Press Exit and select Yes for saving the settings

- Your back to Antenna Setup screen

- Here you have Three choices:

  - Channel Scan

  - Auto Scan

  - Blind Scan

- Select Auto Scan

- Press OK

- You are in the Auto Scan window and you see the Bird(Sat.) you selected and on the top of your screen are three options:

  - Search option

  - Network search

  - Scan Start

- You don’t change nothing here, go down and select Scan Start and press OK

- You do this for all the Birds (Sat.) you have

- Wait for Autoroll and watch TV

- Loading your BIN using USB Jump Drive

- Now its time to download your BIN

- Insert the Memory Stick in your PC and transfer the bin in. Then insert in your STB and in the Main Menu select System information and press OK

- Select Receiver Upgrade and press OK

- Go down to the bottom of the window, select USB and press OK

- A new window opens with some Korean files, years, etc.

- In the bottom of the window, it is your bin selects and presses OK
VERY IMPORTANT. DO NOT TURN THE POWER OFF

- Press Menu key

- Select Installation and press OK

- Select Antenna Setup and press OK

- Select your Sat.

- Here if you have a DISEqC switch enter the port #, Motor or Legacy SW

- No LNB Type need to select (from new, you have only two selection Standard and Universal).  STB will select your LNB Automatic

- Press Exit and select Yes for saving the settings

- Your back to Antenna Setup screen

- Here you have Three choices:

  - Channel Scan

  - Auto Scan

  - Blind Scan

- Select Auto Scan

- Press OK

- You are in the Auto Scan window and you see the Bird(Sat.) you selected and on the top of your screen are three options:

  - Search option

  - Network search

  - Scan Start

- You don’t change nothing, go down and select Scan Start and press OK

- You do this for all the Birds (Sat.) you have

- Wait for Autoroll and watch TV

By: KidRomeo2