Also, How to JTAG 3500? - Instructions Instruction how to Jtag 3500S. 100% Working. 1. Connect cable to receiver and computer and turn on the power switch on back of receiver 2. Start Wall_v2.0 and click on OK on the Warning Message 3. Click on Reset uP and close the waning message again. do this 2 more times closing the warning message each time 4. Leave the wall program open and do not close it 5. Start the Unlock program 5. Under the IRD Model select STi55xx_2M 6. Click on Flash Programming a window will open up just hit OK to continue on 7. Click on UNLOCK and wait until it completes 8. Close that window and close the unlocker program but leave Wall open 9. Start JKEYS and under the IRD Model select STi55xx_2M 10. Click on Flash Programming a window will open up just hit OK to continue on 11. Click on ERASE (just under the work Full) 12. Wait for it to finish erasing 13 Now hit Program and select the correct bin file either 2700A or the 3500S depending on which one you are fixing. 14. After it has completed programming you can hit OK, close down Jkeys and wall and turn off the receiver from the back 15 Unhook the JTAG cable and turn the receiver back on and it should start up. 16. Receiver now show B-70. Open GTRomloaderv35.01_pansat3500S 17. Select bin Pansat_3500S_060526_122_api 18. Click Download (This will get rid of B-70 and now receiver will show "ON") 19. Open GTRomloaderv35.01_pansat3500S 20. Select bin boot_ltc80__06_1019 and hit Download (This will upgrade to B-80) 21. Receiver now shoe B-80 Select bin Pansat 3500S_061217_B80_v02_api 22. Click Download. Now your receiver should be display "ON" 23. Load the latest Bin for Pansat 3500S 24. ENJOY TV

- conecta el cable  a tu reciver y  a la PC y enciende el reciver con el switch de atras

- Star wall-v2.0 y click  en OK cuando aparezca el mensaje de “warning”….

- Click en Reset Up y close el mensaje de “warning  otra vez. Haslo 2 veces mas,  cerrando el mensaje de warnin g cada vez que este mensaje aparezca.

- 4.Deja abierto el  Programa Wall y no lo cierres……………………

- Abajo del IRD model selecciona STi55xx-2M………………………..

- Click en Flash Programing y una Window se abrira y click  OK para continuar….

- Click en Unlock  y espera hasta que este termine ………………………

- Cierra esa window y cierra el unlocker program pero deja abierta Wall (no cierras wall)

- Star JKEYS y bajo el IRD model selecciona STi55xx-2M……….

- Click en Flash Programming y una window se abrira , click OK para continuar.

- Click en ERASE(abajito de la palabra  Full)

- Espere a que este termine de borrar………………

- Ahora Click en Program y selecciona el correcto bin file (2700A o 3500SD ,dependiendo cual reciver estes reparando………………….

- Despues que este a completado el “Programming) click OK  y cierra JKEYS y Wall y apaga el reciver con el switch de atras………………………….

- Desconecta el Jtag cable y enciende tu reciver, este  revivira…..un decir….

- El reciver ahora mostrara B-70, Open Gtromloader v35.01-pansat3500sd..

- Seleciona  bin pansat3500S-060526-122-api.

- Click en download(b-70 desaparezera y el reciver mostrara  ON.

- Open GtRomloaderv35.01 de pansat3500sd………

- Selecciona bin Boot-ltc80-061019 y click download, el reciver sera upgrade a b80………………………..

- El reciver ahora mostrara B80, ahora selecciona bin Pansat3500S-061217-B80-V02-api..

- click download. Al finalizar tu reciver dira ON…………………

- Ahora download el ultimo bin de pansat3500SD…

- disfruta TV…..