| *WARNING* Settings may vary between sets, but this is what worked for me. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Component inputs |  |  |  |  |  |  |  |  |  |  |
|  | Standard | Dynamic | Theater | Game |  |  |  |  |  |  |  |
| Tint | 0 | 3 | 0 | 1 |  |  |  |  |  |  |  |
| color | 4 | 0 | 7 | 10 |  |  |  |  |  |  |  |
| picture | -7 | 10 | 3 | -10 |  |  |  |  |  |  |  |
| brightness | 12 | 10 | 13 | 12 |  |  |  |  |  |  |  |
| detail | 0 | 5 | 0 | 0 |  |  |  |  |  |  |  |
| Color temp | low | high | Low | High |  |  |  |  |  |  |  |
|  | HDMI input |  |  |  |  |  |  |  |  |  |  |
|  | Standard | Dynamic | Theater | Game |  |  |  |  |  |  |  |
| Tint | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |
| color | 4 | -3 | 7 | 4 |  |  |  |  |  |  |  |
| picture | -7 | 10 | 3 | -10 |  |  |  |  |  |  |  |
| brightness | 12 | 10 | 13 | 14 |  |  |  |  |  |  |  |
| detail | 0 | 5 | 0 | 0 |  |  |  |  |  |  |  |
| Color temp | Low | High | Low | High |  |  |  |  |  |  |  |
| Digital Noise correction: Low for all settings |  |  |  |  |  |  |  |  |  |  |  |
|  | Color Management: |  |  |  |  |  |  |  |  |  |  |
| I use OFF for most settings. I left it ON on my "Game" setting. |  |  |  |  |  |  |  |  |  |  |  |
| ON- it boosted the greens quite a bit, making grass and astroturf really JUMP out. I like it turned on for HD football and videogames. |  |  |  |  |  |  |  |  |  |  |  |
| ON- threw off the reds slightly, and lightened up the blues to some degree. Turned yellows slightly orangish. |  |  |  |  |  |  |  |  |  |  |  |
| OFF- was a smoother and more acurate look with less color bleeding on the edges. |  |  |  |  |  |  |  |  |  |  |  |
| OFF- was the only way to get an accurate reading with the test equipment |  |  |  |  |  |  |  |  |  |  |  |
|  | Dynamic Gamma: |  |  |  |  |  |  |  |  |  |  |
| I left it ON for all settings, I switch it OFF when watching SD TV. |  |  |  |  |  |  |  |  |  |  |  |
| ON- darkened the blacks, but you lose a slight amount of detail in the dark areas. |  |  |  |  |  |  |  |  |  |  |  |
| OFF- a very soft "washed out" look. Dark areas had more detail, but mainly due to blacks turning a dark gray, making them more visible. |  |  |  |  |  |  |  |  |  |  |  |
| OFF- made SD TV look better. The softer look hid the flaws and graininess of the analog signal much better. |  |  |  |  |  |  |  |  |  |  |  |
|  | Final judgment: |  |  |  |  |  |  |  |  |  |  |
| Theater mode using HDMI was about as close to perfect (D6500k) as this TV will get. |  |  |  |  |  |  |  |  |  |  |  |
| Grayscale was corrected by adjusting brightness and picture, and leaving Dynamic Gamma On. |  |  |  |  |  |  |  |  |  |  |  |
| Blues are dead on in all settings |  |  |  |  |  |  |  |  |  |  |  |
| Reds are dead on with HDMI, about 95% correct with component. |  |  |  |  |  |  |  |  |  |  |  |
| Greens are 95% with HDMI, 85% with component. |  |  |  |  |  |  |  |  |  |  |  |
| Green is the least crucial of the 3 colors, so variance with it is quite acceptable |  |  |  |  |  |  |  |  |  |  |  |
| Overall brightness, color, and smoothness was much better than DLP and LCD sets that "Dave" has worked with |  |  |  |  |  |  |  |  |  |  |  |
| Pixel spacing was tigher than LCD and DLP, resulting in a smoother but less sharp looking picture. |  |  |  |  |  |  |  |  |  |  |  |